Video Demonstrating How To Install and Operate the WOCD

WOCD Screen by Screen Installation Instructions

Intro Video

This is how your WOCD will work after you install it.  The WOCD is made for either the left or right hand.  A left-handed WOCD fits with a window with the lock bar on the left and the hinge on the right when viewed from the interior looking out.

The WOCD is designed to keep your window from opening more than 4” and be difficult for a child 5 years old and younger from opening.    To release the window, push the slider down, then gently push the red pin out of the bottom receiver while slowly winding your window closed.

The videos that follow, show you how to install and operate the WOCD..

AI Change

“This is how your WOCD—Window Opening Control Device—works once it’s installed.

The WOCD is designed for use on either the left or right side of your window.
A left-handed WOCD fits windows with the lock bar on the left and the hinge on the right—when viewed from the inside looking out.

This device helps keep your window from opening more than 4 inches, and it's specifically designed to be difficult for children aged 5 and under to open.

To release the window:
Push the slider down, then gently push the red pin out of the bottom receiver as you slowly close the window.

It’s that simple—safe, secure, and easy to use.”

- 1st Video Identify hinge cavity where WOCD will be installed

Find the hinge cavity of your window by first finding the hinge, then follow the hinge cavity it sits in all the way to the vertical part of the window frame.  The bottom receiver will be secured into this hinge cavity.   The top clasp will be secured to a channel in the sash that meets the bottom receiver in the hinge cavity.

AI Change

“To begin installation, locate the hinge cavity of your window.

Start by finding the hinge itself—then follow the cavity it sits in, all the way to where it meets the vertical part of the window frame.

This is where you’ll secure the bottom receiver of the WOCD.

Next, identify the channel in the sash—the moving part of the window—that aligns with the hinge cavity.

This is where the top clasp will be installed, so it meets the bottom receiver when the window is closed.”

- Tape install template to window frame

Take the template from your package and tape it to the bottom corner of the window tight against the vertical hinge cavity.

3. Drill holes through the template  (3A & 3B)

Starting with the bottom receiver, use a 1/8“drill bit to drill holes through the centre of the two bottom marks on the template.

Now close the sash against the template, then move to the outside of the window and drill two holes through the centre of the top two template marks.

4. Attach the bottom receiver

Attach the bottom receiver to the window frame using the 2 pan head screws provided starting with the higher pre-drilled hole (do not over tighten).

AI Change

Using the provided pan head screws, attach the bottom receiver to the window frame.
Start with the higher hole, and make sure not to overtighten the screws.

5. Finetuning the bottom receiver

The hook arm should align with the outer edge of the sash so that the sash pushes the hook up as it closes.  If needed, gently bend the latch to achieve proper alignment.

AI Change

The hook arm should align with the outer edge of the sash.
This allows the sash to push the hook up as it closes.
If needed, gently bend the latch to fine-tune the alignment.

6. Install the top clasp arm

Using the predrilled holes, attach the top clasp arm with the 2 pan head screws provided into the sash starting with the bottom hole which is in the middle of the shim.  .  If the clasp arm is not being held up by the spring, place the clasp arm onto the spring so that the arm is held forward by the spring to allow it to connect to the bottom receiver in the next step.

AI Change

Using the pre-drilled holes, attach the top clasp arm to the sash with the two provided pan head screws.
Start with the bottom hole—it’s centered on the shim.

If the clasp arm isn’t held forward by the spring, place it onto the spring so it stays forward and can connect to the bottom receiver.

7. Finetuning the top clasp arm

Once the hook arm is aligned with the window, the top clasp arm should meet it at the centre of the pin.  If necessary, bend the top arm to ensure connection.  Repeat with small adjustments until proper alignment is achieved.

AI Change

With the hook arm aligned, the top clasp arm should meet it at the center of the pin.
Bend the arm gently as needed to ensure a snug, accurate fit.
Make small adjustments until perfect alignment is achieved.

8. Connecting the bottom latch with top arm

If the latch is not already open, slide the red sleeve (with arrow) down to open it.  Wind your sash / window in to connect the pin with the latch.  Open and close you window a few times to verify latch aligns with the pin.

The WOCD must be restrict the window to a 4” opening or less.  If the opening exceeds 4”, move the clasp arm upward (1/4” up will reduce the opening roughly ½ “).

AI Change

If the latch is not already open, slide the red sleeve (with the arrow) down.
Gently wind the sash in so that the pin connects with the latch.
Open and close the window a few times to verify a smooth connection.

The WOCD should limit the window opening to 4 inches or less.
If the opening exceeds this, move the clasp arm upward—just ¼ inch up reduces the opening by about ½ inch.

9. Releasing the WOCD

Open the window to its maximum opening not exceeding 4” and slide the red sleeve down to the bottom of the receiver to allow the latch to open.

While closing the window, gently push the round pin. As the pin passes over the latch opening, gently push it outward to release it from the clasp and open the window.

The WOCD Device is now released and will re-engage in the locked position when the window is closed.

AI Change

Open the window to its maximum allowed opening (no more than 4 inches).
Slide the red sleeve down to the bottom of the receiver to unlock the latch.

As you slowly close the window, gently push the round pin outward as it passes over the latch to fully release it.
Once the window is reopened, the WOCD will automatically re-engage when closed again.

9. Apply the WARNING label on window frame

Place the window decal on the sash in a visible place.

The small temporary label / hang tag in the package should be hung around the WOCD if your house still requires final code inspection.

AI Change

Place the warning decal on the sash in a visible location.

If your home still requires a final code inspection, hang the temporary tag around the WOCD for easy identification.